Field Phase Report for ESQUIF-99 (3 campaigns from June 25 to July 18 1999)

Table 1: Times airborne (take-off – landing in UTC):

| Flight (Date) | morning flight | afternoon flight | remarks (details below) |
| --- | --- | --- | --- |
| F990625A | 08:12 – 10:22 |  |  |
| F990625B |  | 12:21 – 16:21 |  |
| F990702A | 07:32 – 09:54 |  | no C4 |
| F990702B |  | 12:22 – 16:34 | no C4 |
| F990716A |  | 11:35 – 15:36 | no NOxTOy after 3 h |
| F990717A | 07:54 – 1005 |  |  |
| F990717B |  | 14:16 – 18:24 | late, but, "best" O3-plume found |
| F990718A | 08:00 – 10:21 |  | remakable reservoir layer |
| F990718B |  | 12:16 – 16:15 |  |

Table 2: Summary of remarks on the flight log:

F990625A:	E-wind, blue sky, shallow haze, no technical remarks – no problems

F990625B:	take-off delayed due to heavy traffic – no other remarks

F990702A:	weak S-wind, blue sky, 1/8 Ci aviaticus fract., shallow haze
GC without CO2-cooling, hence perhaps no C4

F990702B:	wind variable, bue sky, Ci aviaticus above Orly,
GC still without CO2

F990716A:	5-7/8 Ci, some Ac, <1/8 low Cu in the morning, but, increasingly sunny now;
NOxTOy without Luminol after 3 h of flight

F990717A:	weak SE-wind, blue sky, some Ci aviaticus, shallow layer of haze

F990717B:	still blue sky with Ci av., wind calm with intensive thermal plumes;
departure severlely delayed due to pan before;
very clear plume found north of Paris

F990718A:	blue sky, some Ac in the east, decreasing

F990718B:	<1/8 flat Cu, changing Ac; ideal conditions, prolongation of flight would have been
desirable, but, fuel bacame low. Thunderstorms after 21 h CEST (19 h UTC).

Table 3: Mesured parameters on board of MetAir's aircraft "ECO-Dimona HB-2335" for ESQUIF-99

parameter	instrument/method	range	resolution	precision /	calibration
		from..to	parameter / time	accuracy	or checks

air temperature	Meteolabor thermocouple	-50..50 oC	0.1 oC / 10 Hz	0.1 / 0.5 oC	ice-water/mercury
dewpoint	Meteolabor dewopoint mirror	-50..50 oC	0.1 oC / 2 s	0.1 / 0.5 oC	psychrometer

5 pressures (1 absolute, 4 diff.) for 	capacitve sensors from Keller Druckmesstechnik	300..1300; 0..50 hPa	0.02 hPa / 10 Hz	0.1 / 0.5 hPa	factory calibrated
flow angles	differences of pairs (left/right, top/bottom)	-20..20°	0.1°/0.1 s	0.1°/0.5°	wind residuals
true airspeed,	calculated (p,T,u)	10..70 m/s	0.1 m/s / 10 Hz	0.2 / 0.5 m/s	wind residuals
and pressure altitude	integrated (p,T,u)	0..7000 mMSL	1 m / 4 s	3 / 10 m	mountaintops

height above ground	radar altimeter TERRA	15..780 m	1 m / 1 s	1 / 5 m	against pressure altitude

position (x,y,z)	Trimble TANS Vector	worldwide	7 m / 1 s	20 / 100 m*	fixpoints from map
ground speed (vx, vy)		do.	0.1 m/s / 1 Hz	0.1 / 0.5 m/s	statically at ground
attitude angles (azimuth, pitch, and roll)		do.	0.1° / 3..10 Hz	0.1 / 0.5°	statically at ground
vertical acceleration	Kistler/DLR	-1..3 g	0.01 m/s2	0.01 m/s2	integral tied to z(p)

3-d-wind (x,y,z)	post flight processing from parameters above	0.5..30 m/s	0.5 m/s / 3 Hz	0.5 / 1.0 m/s	wind during maneuvers

aerosols (>0.3 and >0.5 m)	MetOne Laser Particle Counter	0..200 n/cm3	1 n/cm3 / 1 s	1 / 10 n/cm3	factory calibration

O3 (slow) ++	PSI / UV absorption	2 ppb .. 1 ppm	1 ppb / 4 s	1 / 2 ppb	calibration gas+
 
NO2, NOx, NOy, HNO3, PAN, Ox++	6-channel Luminol-detector with converters	0.5..500 ppb	0.1 ppb / 1..5 s	0.5 ppb / 20 %	NO2 calibration gas

speciated hydrocarbons (C4..C10)	Gaschromatograph Airmotec HC-1010	10 ppt..10 ppb	10 ppt / 10'	10 ppt / 50 ppt or 20 %	calibration gas
list of substances in table 2

CO2	open path infrared absorption (NOAA IRGA)	adjustable	0.1 ppm / 10 Hz	1 ppm**	laboratory, and from profiles
H2O	open path infrared absorption (NOAA IRGA)	adjustable	0.01 g/kg / 10 Hz	0.1 g/kg	against dewpoint mirror

flight track F990625A.POS

flight track F990702A.POS

# Flight patterns

The flight patterns consisted of circumferential flights around the city of Paris with a radial of about 30 km, and at least one vertical profile up to 1700 mMSL (on the radial) or up to 3000 mMSL 50 km away from the city (due to air space regulations / air traffic control).

During morning flights, the pattern was flown once. During afternoon flights, the pattern was flown twice, plus (sometimes) an excursion towards the lee-side.

flight track F990625B.POS

flight track F990702B.POS

flight track F990716A.POS

flight track F990717A.POS

flight track F990718A.POS

Instrumental problems

No severe problems known until know. All raw data was quick-look-checked in the field.

Minor problems listed above:

CO2-flux was obstructed in the gaschromatograph on July-2. Therfore, the trap could not be cooled, and C4, ev. also part of C5 will not be separated.

The IRGA showed excessive noise (reason discussed later by e-mail with A.Volz-Thomas). Therefore, the CO2-data will partly be useless, and fast water-vapour measurements will not be reliable. However, CO2 was not a key parameter, and moisture is measured both precise, and fast (up to 1 Hz) with the dewpoint mirror.

The NOxTOy behaved quite well and could be calibrated quite frequently. Especially NO2, NOx, and Ox seem to be reliable, and stable.

flight track F990717B.POS

flight track F990718B.POS

The Mo-Converter for NOy, and NOy minus HNO3 (channels 3, and 4) showed large memories (variable zero offset). Channel 4 quit functioning during the end of the campaign. We will have to wait until the data is processed before we can judge the quality of the NOy-data. However, if it is ok, it is really NOy (not NOw), because the inlet was not glass, but PFA, and was heated to above 50 °C.

There is no reason to believe, that the PAN data has problems (channel 5: NOx + PAN worked well), but, final judgement will also only be possible after data processing.

It is for sure, that both NO2, and Ox are of good quality.

## Further treatment of data/preliminary data

Until February 14, most of the flights are finally treated and distributed. When questions arise during the analysis of the data, or errors are detected, please contact MetAir (metair@zugernet.ch).

*	depending on operation mode (SA) of the satellites; eventually we will have dGPS for better accuracy

+	+	The signals of the two monitors are combined to get both fast and stable measurements, overlapping in the range between about 30, and 600 seconds

*	* 	some problems during whole summer 1999